INTERNET OF THINGS lab. Record

Subject Code:

| Name: | RIDDHIMA BANWALE |
| --- | --- |
| Reg. No : | FET/BDS/2022-26/008 |
| Course: | B.TECH |
| Semester: | 5th Sem |
| Faculty: | Dr. BISWAJIBAN MISHRA |

| Remarks |  |
| --- | --- |
| Signature |  |

SRI SRI UNIVERSITY

Bidyadharpur, Cuttack, Odisha

| Sl. No. | Date | Experiment/Case Study | Page No. | Remark |
| --- | --- | --- | --- | --- |
| 1 | 14/08/2024 | Blinking the inBuilt LED |  |  |
| 2 | 22/08/2024 | Blinking an external LED |  |  |
| 3 | 29/08/2024 | Using DHT sensor |  |  |
| 4 | 12/09/2024 | Using Mosquitto MQTT (Pub-Sub) |  |  |
| 5 | 19/09/2024 | Building a web app using Node-Red to fetch DHT sensor data and display it on the web app dashboard |  |  |
| 6 | 26/09/2024 | Working with ultrasonic sensors. |  |  |
| 7 | 03/10/2024 | Use of ESP32, upload code on ESP 32 to blink onboard LED. |  |  |
| 8 | 17/10/2024 | Using breadboard |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |
| 16 |  |  |  |  |

LAB 1 : Blinking the inBuilt LED

void setup() {

pinMode(LED_BUILTIN, OUTPUT);

}

void loop() {

digitalWrite(LED_BUILTIN, HIGH);

delay(1000);

digitalWrite(LED_BUILTIN,LOW);

delay(1000);

}

# Wokwi 1

LAB 2 : Blinking an external LED

#define led_pin 11

void setup() {

pinMode(led_pin, OUTPUT);

}

void loop() {

digitalWrite(led_pin, HIGH);

delay(1000);

digitalWrite(led_pin,LOW);

delay(1000);

}

## wokwi 2

Lab 3 : Using DHT sensor

#include <DHT.h>

#define pin 7

#define DHTTYPE DHT22

DHT dht(pin, DHTTYPE);

float humid, temp;

void setup() {

Serial.begin(9600);

dht.begin();

}

void loop() {

delay(200);

humid = dht.readHumidity();

temp = dht.readTemperature();

Serial.print("Humidity:");

Serial.print(humid);

Serial.print("%  Temperature: ");

Serial.print(temp);

Serial.println("°C");

delay(1000);

}

#Wokwi 3

Lab 4: Using Mosquitto MQTT (Pub-Sub):

Starting Mosquitto MQTT:

Setting up Publisher & Sending Message:

Setting up Subscriber & Receiving Message:

The mosquitto server is downloaded and implemented in the UNIX terminal of my laptop.

There are 3 photos of which 1 of each represent the part of an mqtt (a broker and client(subscriber and publisher )):

1. Start the mosquitto server by the typing the code [brew services start mosquitto].

2. in another terminal we made a subcriber to the [lab_example] topic using the code [mosquitto_sub -h localhost -t lab_example]

3. in another terminal i made a publisher to the topic [lab_example] using [mosquitto_pub -h localhost -t lab_example -m "\\I am a publisher to this topic\\"]

4. stop the server with the code [brew services stop mosquitto]

## We can also allow subscriber to subcribe to our topic using our network .

Lab 5: Building a web app using Node-Red to fetch DHT sensor data and display it on the web app dashboard

Installing & Initialising node red:

- Open node.js > npm install node-red-dashboard

- [postinstallation] > elevated cmd: node-red

In client application, browsed localhost:1880 [ accessing node red]:

- Inside the nodered window, a flow was created w/ the nodes as:

> serial-in ( arduino uno r3 board )

> debugger

> dht function

> 2 gauges (humidity & temperature)

- Serial in node: configured it to read from the correct serial port where my arduino is connected (e.g., com7) > set the baud rate to 9600.

- Configure the dht function as:

var m = msg.payload.split(',');

if (m.length === 2) {

var h = { payload: parsefloat(m[0]) };

var t = { payload: parsefloat(m[1]) };

return [h, t];

} else {

return null; }

- Adjusting Gauge Nodes:

Humidity:

- Title as “ Humidity ”.

- Value format as ‘ {{value}}% ’.

- Range Value: 0 ~ 100 %.

Temperatue:

- Title as ' Temperature '.

- Value format as ‘ {{value}}°C ’.

**Ensure that Humidity & Temperature are in the same group

Deployment:

- Uploaded DHT11 /22 Sketch to the Arduino Board through its IDE:

#include <dht.h>

#define dhtpin 3

#define dhttype dht11

dht dht(dhtpin, dhttype);

void setup() {

serial.begin(9600);

dht.begin();

}

void loop()  {

float h = dht.readhumidity();

float t = dht.readtemperature();

if (isnan(h) || isnan(t)) {

serial.println("failed to read from dht sensor!");

}

else {

serial.println(string(h) + "," + string(t));

}

delay(2000);

}

- After uploading this sketch, close the IDE.

- Deploy the flow in NodeRED.

- Check the Dashboard in the upper-right corner, for the Humidity and Temperature Gauge.

OUTPUT ON THE DASHBOARD:

LAB 6: Working with ultrasonic sensors .

## Measuring distance of a somethings using ultrasonic sensors

CODE:

#include <DHT.h>

#define PIN_TRIG 9

#define PIN_ECHO 8

void setup() {

Serial.begin(9600);

pinMode(PIN_TRIG, OUTPUT);

pinMode(PIN_ECHO, INPUT);

}

void loop() {

digitalWrite(PIN_TRIG, HIGH);

delayMicroseconds(10);

digitalWrite(PIN_TRIG, LOW);

int duration = pulseIn(PIN_ECHO, HIGH);

float distanceCm = duration / 58.0;

Serial.print("Distance in CM: ");

Serial.println(distanceCm);

delay(1500);

}

Lab#7 Use of ESP32, upload code on ESP 32 to blink onboard LED

Step 1: install esp32 board from the board manager

Step 2 : choose the esp32 board from the port

Step 3: compile and upload the code .

Code 1 : blinking inbuilt led

#define LED_PIN 2

void setup() {

pinMode(LED_PIN, OUTPUT);

}

void loop() {

digitalWrite(LED_PIN, HIGH);

delay(1000);

digitalWrite(LED_PIN, LOW);

delay(1000);

}

Code 2 : Finding nearby using esp32

#include "WiFi.h"

void setup() {

Serial.begin(9600);

WiFi.mode(WIFI_STA);

WiFi.disconnect();

delay(100);

Serial.println("Setup done");

}

void loop() {

Serial.println("Scan start");

int n = WiFi.scanNetworks();

Serial.println("Scan done”);

if (n == 0) {

Serial.println("no networks found");

} else {

Serial.print(n);

Serial.println(" networks found");

Serial.println("Nr | SSID                             | RSSI | CH | Encryption");

for (int i = 0; i < n; ++i) {

Serial.printf("%2d", i + 1);

Serial.print(" | ");

Serial.printf("%-32.32s", WiFi.SSID(i).c_str());

Serial.print(" | ");

Serial.printf("%4ld", WiFi.RSSI(i));

Serial.print(" | ");

Serial.printf("%2ld", WiFi.channel(i));

Serial.print(" | ");

switch (WiFi.encryptionType(i)) {

case WIFI_AUTH_OPEN:            	     Serial.print("open"); break;

case WIFI_AUTH_WEP:             	     Serial.print("WEP"); break;

case WIFI_AUTH_WPA_PSK:        	     Serial.print("WPA"); break;

case WIFI_AUTH_WPA2_PSK:                   Serial.print("WPA2"); break;

case WIFI_AUTH_WPA_WPA2_PSK:  	     Serial.print("WPA+WPA2"); break;

case WIFI_AUTH_WPA2_ENTERPRISE:    Serial.print("WPA2-EAP"); break;

case WIFI_AUTH_WPA3_PSK:       	     Serial.print("WPA3"); break;

case WIFI_AUTH_WPA2_WPA3_PSK:        Serial.print("WPA2+WPA3"); break;

case WIFI_AUTH_WAPI_PSK:        	     Serial.print("WAPI"); break;

default:                        			     Serial.print("unknown");

}

Serial.println();

delay(10);

}

}

Serial.println(“");

WiFi.scanDelete();

delay(5000);

}

Lab Exercise 8: Use of Breadboard

Objective: Learn basic breadboard interfacing and circuit assembly.

Components: Breadboard, jumper wires, resistors, LEDs.

In this project, we did the process to blink three LEDs using for loop. The three LEDs will light up one after the other.

Expected Outcome: Familiarity with breadboard layout and component arrangement.