HANDS ON EXPERIMENT-G5

The 7 segment Display

EXPERIMENT 1: Printing Numbers to the Single Digit Display

CODE:

const int segmentPins[8] = {2, 3, 4, 5, 6, 7, 8};

const int commonCathodePin = 9;

const byte numbers[11] = {

B11111100,

B01100000,

B11011010,

B11110010,

B01100110,

B10110110,

B10111110,

B11100000,

B11111110,

B11110110

};

void setup() {

for (int i = 0; i < 7; i++) {

pinMode(segmentPins[i], OUTPUT);

}

pinMode(commonCathodePin, OUTPUT);

}

void loop() {

for (int i = 0; i < 10; i++)

{

displayNumber(i);

delay(1000);

}

}

void displayNumber(int num) {

for (int i = 0; i < 8; i++) {

digitalWrite(segmentPins[i], HIGH);

}

for (int i = 0; i < 8; i++) {

if (bitRead(numbers[num], i) == LOW) {

digitalWrite(segmentPins[7-i], LOW);

}

}

}

Link of the Project: HANDS ON EXP 1

EXPERIMENT 2: Printing Numbers using the SevSeg library with a                      4-digit display. It displays a counter that counts up, showing Deci seconds.

CODE:

#include "SevSeg.h"

SevSeg sevseg;

void setup() {

byte numDigits = 4;

byte digitPins[] = {2, 3, 4, 5};

byte segmentPins[] = {6, 7, 8, 9, 10, 11, 12, 13};

bool resistorsOnSegments = false;

byte hardwareConfig = COMMON_ANODE;

bool updateWithDelays = false;

bool leadingZeros = false;

bool disableDecPoint = false;

sevseg.begin(hardwareConfig, numDigits, digitPins, segmentPins, resistorsOnSegments,

updateWithDelays, leadingZeros, disableDecPoint);

sevseg.setBrightness(90);

}

void loop() {

static unsigned long timer = millis();

static int deciSeconds = 0;

if (millis() - timer >= 100) {

timer += 100;

deciSeconds++;

if (deciSeconds == 10000) {

deciSeconds=0;

}

sevseg.setNumber(deciSeconds, 1);

}

sevseg.refreshDisplay();

}

Link of the Project: HANDS ON EXP 2

EXPERIMENT 3: Digital Dice Using 7 digit display

CODE:

#define resett 15

#define dice 14

char digit[6]={0x02, 0x79, 0x24, 0x30, 0x19, 0x12};

int pin[7]={6,5,4,3,2,1,0};

void setup()

{

for(int i=0;i<7;i++)

pinMode(pin[i], OUTPUT);

pinMode(dice, INPUT);

pinMode(resett, INPUT);

digitalWrite(dice, HIGH);

digitalWrite(resett, HIGH);

int temp=0x40;

for(int i=0;i<7;i++)

{

int temp1=temp&0x01;

digitalWrite(pin[i], temp1);

temp=temp>>1;

}

delay(1000);

}

void loop()

{

int temp=rand();

if(digitalRead(dice)==0)

{

int k=temp%6;

temp=digit[k];

wait();

for(int i=0;i<7;i++)

{

int temp1=temp&0x01;

digitalWrite(pin[i], temp1);

temp=temp>>1;

}

delay(200);

}

if(digitalRead(resett)==0)

{

temp=0x40;

for(int i=0;i<7;i++)

{

int temp1=temp&0x01;

digitalWrite(pin[i], temp1);

temp=temp>>1;

}

}

}

void wait()

{

for(int m=0;m<10;m++)

{

for(int k=0;k<6;k++)

{

int ch=digit[k];

for(int l=0;l<7;l++)

{

char tem2=ch&0x01;

digitalWrite(pin[l], tem2);

ch=ch>>1;

}

delay(50);

}

}

}

Link of the Project: HAND ON EXP 3