INTERNET OF THINGS Lab. Record

Subject Code:____________________

| Name: | Gayathri Reddy Epur |
| --- | --- |
| Registration Number: | FET-BAML-2022-26-018 |
| Course: | IOT |
| Semester: | 5th |
| Faculty: | Biswajeeban Mishra Pritam Nanda |

| Remarks |  |
| --- | --- |
| Signature |  |

SRI SRI UNIVERSITY

Bidyadharpur, Cuttack, Odisha.

Index

| Sl. No. | Date | Experiment/Case Study | Page No. | Remark |
| --- | --- | --- | --- | --- |
| 1 | 14-8-2024 | Blink Inbuilt Led of your Arduino Board |  |  |
| 2 | 22-8-2024 | Blink an external LED |  |  |
| 3 | 29-8-2024 | Using DHT sensor |  |  |
| 4 | 12-9-2024 | A case study on a communication Protocol: MQTT |  |  |
| 5 | 19-9-2024 | Building a web app using Node-Red to fetch DHT sensor data and display it on the web app dashboard |  |  |
| 6 | 26-9-2024 | Interfacing Ultrasonic Sensor |  |  |
| 7 | 3-10-2024 | Use of Breadboard |  |  |
| 8 | 17-10-2024 | Use of ESP32, upload code on ESP 32 to blink onboard LED |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |

Date: 14-8-2024

Experiment No.:1

Aim:

To learn how to control the built-in LED on an Arduino board using Arduino IDE.

Requirements:

Arduino board (Uno R3), USB cable , Arduino IDE software

Procedure:

- Connect your Arduino board to your computer using a USB cable.

- Open the Arduino IDE software on your computer.

- Write the source code on the software.

- Connect your Arduino board to your computer.

- Select the correct board type and port in the Arduino IDE's Tools menu.

- Click the Upload button in the Arduino IDE to transfer the code to the board.

- The built-in LED on your Arduino board should now blink on for 1 second and off for 5 second.

Snapshots of the experiment:

Date: 22-8-2024

Experiment No.:2

Aim:

To learn how to control the built-in LED on an Arduino board using Arduino IDE.

Requirements:

Arduino board (Uno R3), USB cable , Arduino IDE software, External LED (with two leads), Jumper wires

Procedure:

- Connect your Arduino board to your computer using a USB cable.

- Open the Arduino IDE software on your computer.

- Write the source code on the software.

- Use a jumper wire to connect the anode[+](longer end) of the LED directly to digital pin 4 on the Arduino board.

- Use another jumper wire to connect the cathode[-](shorter end) of the LED directly to the ground pin on the Arduino board.

- Select the correct board type and port & click the upload button.

- The external LED should now blink on and off with a one-second interval.

Snapshots of the experiment:

Date: 29-8-2024

Experiment No.:3

Aim:

To measure temperature and humidity using a DHT11 or DHT22 sensor with an Arduino board.

Requirements:

Arduino board (Uno R3), USB cable ,Arduino IDE, DHT11 or DHT22 sensor, Jumper wires

Understanding DHT Sensor pins:

DHT22 Sensor:                                   DHT11 Sensor:

Procedure:

- Connect your Arduino board to your computer using a USB cable.

- Search for ‘DHT library’ in the Arduino Library Manager and install the ‘DHT sensor library by Adafruit’.

- Now write the source code on the software.

- Compile(click Verify) the code and click the Upload button.

- Open the Serial Monitor in the Arduino IDE.

- The temperature and humidity values will be printed periodically.

Snapshots of the experiment:

Date: 12-9-2024

Experiment No.:4

Aim:

- To understand the MQTT communication protocol.

- To install and configure an MQTT broker (server).

- To write and test MQTT client code to publish and subscribe to messages.

About MQTT:

MQTT (Message Queuing Telemetry Transport) is a lightweight, publish-subscribe messaging protocol designed for constrained devices and low-bandwidth networks. It's particularly well-suited for Internet of Things (IoT) applications.

It’s key components are:

Topic: A hierarchical string that categorizes messages.

Publisher: A device or application that sends messages to a specific topic.

Subscriber: A device or application that receives messages from a specific topic.

Broker: A server that facilitates communication between publishers and subscribers.

Mosquitto is a popular open-source MQTT broker that implements the MQTT protocol.

Procedure:

[Task-A] Installing Mosquitto

On Linux:

- Install and Ubuntu on the device.

- Install Mosquitto and Client Packages :

sudo apt install mosquitto mosquitto-clients

- Enable and Start the Mosquitto Service :

sudo systemctl enable mosquitto sudo

systemctl start mosquitto

On Windows:

- Visit the official Mosquitto download page and download the appropriate version of Mosquitto (generally the .exe installer for 64-bit Windows).

- Complete the installation process

- Run the following command to start Mosquitto in the foreground:

mosquitto.exe -v

The -v flag is for verbose output so that you can see log messages.

[Task-B] Test Mosquitto

Open Two Terminal Windows to test MQTT communication.

Terminal 1(Subscribing):

mosquitto_sub -h localhost -t test/topic

Terminal 2(Publising):

mosquitto_pub -h localhost -t test/topic -m "Hello from Mosquitto!"

Snapshots of the experiment:

Date: 19-8-2024

Experiment No.:5

Aim:

To build a web dashboard in Node-Red to display DHT sensor data.

Requirements:

Arduino board (Uno R3), USB cable, Node-Red environment, DHT11 or DHT22 sensor, Jumper wires

Procedure:

- Open node.js > npm install node-red-dashboard

- [postinstallation] > elevated cmd: node-red

- In client application, browse localhost:1880 to access node red

- Inside the nodered window, a flow was created w/ the nodes as:

  - serial-in ( arduino uno r3 board )

  - debugger

  - dht function

  - 2 gauges (humidity & temperature)

- Configured Serial in node to read from the correct serial port where my Arduino is connected & set the baud rate to 9600.

- Configure the dht function as:

var m = msg.payload.split(',');

if (m.length === 2) {

var h = { payload: parsefloat(m[0]) };

  - var t = { payload: parsefloat(m[1]) };

  - return [h, t];

} else {

return null;

}

- Adjusting two gauge nodes:

  - Humidity Gauge:

    - Set the title to "Humidity".

    - Set the value format to {{value}}%.

    - Set the minimum value to 0 and the maximum to 100.

  - Temperature Gauge:

    - Set the title to "Temperature".

    - Set the value format to {{value}}ºC.

    - Set the minimum value to 0 and the maximum to 100.

- Upload the  dht11 /22 sketch to the arduino board through its ide:

#include <dht.h>

#define dhtpin 3

#define dhttype dht11

dht dht(dhtpin, dhttype);

void setup() {

serial.begin(9600);

dht.begin();

}

void loop()  {

float h = dht.readhumidity();

float t = dht.readtemperature();

if (isnan(h) || isnan(t)) {

serial.println("failed to read from dht sensor!");

}

else {

serial.println(string(h) + "," + string(t));

}

delay(2000);

}

- after uploading this sketch, close the ide.

- deploy the flow in nodered.

- check the dashboard in the upper-right corner, for the humidity and temperature gauge.

Snapshots of the experiment:

Date: 26-9-2024

Experiment No.:6

Aim:

To measure distance using an ultrasonic sensor and display the result on the serial monitor.

Requirements:

Arduino board (Uno R3), USB cable ,Arduino IDE, Ultrasonic sensor (HC-SR04), Jumper wires

Understanding Ultrasonic sensor (HC-SR04) pins:

Procedure:

- Connect your Arduino board to your computer using a USB cable.

- Connect the sensor:

  - Connect the Vcc pin to the 5V pin on the Arduino.

  - Connect the GND pin to the GND pin.

  - Connect the Trig pin to a digital pin.

  - Connect the Echo pin to another digital pin.

- Write the Source code, verify and upload the code to the Arduino board.

- Open the Serial Monitor

- The distance measurement in centimeters is printed periodically.

Snapshots of the experiment:

Date:3/10/2024

Experiment No.:7

Aim:

To Learn basic breadboard interfacing and circuit assembly.

Requirements:

Arduino board (Uno R3), USB cable, Breadboard, jumper wires, resistors, LEDs.

Procedure:

- Connect one leg of each resistor to the 5V power rail.

- Connect the other leg of each resistor to the longer leg (anode) of an LED.

- Connect the shorter leg (cathode) of each LED to a different digital pin on the Arduino (e.g., pins 2, 3, and 4).

- Connect the other leg of each resistor to the ground rail.

- Connect the Arduino to the Breadboard:

  - Power: Connect the 5V pin on the Arduino to the 5V power rail on the breadboard.

  - Ground: Connect the GND pin on the Arduino to the ground rail on the breadboard.

- Write the Source Code in Arduino IDE or Wokwi.

- Connect the Arduino to your computer via USB.

- Open the Arduino IDE, select the correct board and port, and upload the code.

- The LEDs should now blink in a specific pattern as defined by the code.

Snapshots of the experiment:

Date:17/10/2024

Experiment No.:8

Aim:

To Use the ESP32 microcontroller to blink its onboard LED.

Requirements:

ESP32 board, Arduino IDE, USB cable

Procedure:

- Go to File > Preferences in the Arduino IDE.

- In Additional Boards Manager URLs, paste: https://dl.espressif.com/dl/package_esp32_index.json and click OK.

- Open Boards Manager (Tools > Board > Boards Manager) and search for ESP32 by Espressif Systems. Click Install.

- Download and install the CP210x USB-to-UART Bridge VCP Driver to enable communication between your computer and the ESP32 board.

- After restarting, select ESP32 Dev Module from the Tools > Board menu.

- Choose the correct COM port for your ESP32 (e.g., COM7).

- Press and hold the BOOT button while uploading the code.

- Release the BOOT button once the upload begins (when you see "Connecting...").

- Use the following code in the Arduino IDE to blink the onboard LED.

#define LED_PIN 2

void setup() {

pinMode(LED_PIN, OUTPUT);

}

void loop() {

digitalWrite(LED_PIN, HIGH);

delay(1000);

digitalWrite(LED_PIN, LOW);

delay(1000);

}