IoT & Automation Lab Record

| Name: | Aditya Narayan Purohit |
| --- | --- |
| Reg. No : | FET/BDS/2022-26/006 |
| Course: | B.TECH |
| Semester: | 5th Sem |
| Faculty: | Dr. BISWAJIBAN MISHRA |

| Remarks |  |
| --- | --- |
| Signature |  |

SRI SRI UNIVERSITY

Bidyadharpur, Cuttack, Odisha

| Sl. No. | Date | Experiment/Case Study | Page No. | Remark |
| --- | --- | --- | --- | --- |
| 1 | 14/08/2024 | Blinking the inBuilt LED |  |  |
| 2 | 22/08/2024 | Blinking an external LED |  |  |
| 3 | 29/08/2024 | Using DHT sensor |  |  |
| 4 | 12/09/2024 | Using Mosquitto MQTT (Pub-Sub) |  |  |
| 5 | 19/09/2024 | Building a web app using Node-Red to fetch DHT sensor data and display it on the web app dashboard |  |  |
| 6 | 26/09/2024 | Working with ultrasonic sensors. |  |  |
| 7 | 03/10/2024 | Use of ESP32, upload code on ESP 32 to blink onboard LED. |  |  |
| 8 | 17/10/2024 | Using breadboard |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |
| 16 |  |  |  |  |

Lab-1

Lab Exercise: Blinking the In-Built LED of Arduino Board

Objective: Familiarize with Arduino programming and syntax by blinking the onboard LED.

Components: Arduino Uno.

Procedure:

- Open Arduino IDE and connect Arduino to your computer.

- Write a basic sketch using the pinMode() and digitalWrite() functions to blink the onboard LED on Pin 13.

- Upload the code and observe the LED.

void setup() {

pinMode(LED_BUILTIN, OUTPUT);

}

void loop() {

digitalWrite(LED_BUILTIN, HIGH);

delay(1000);

digitalWrite(LED_BUILTIN, LOW);

delay(1000);

}

Expected Outcome: The onboard LED should blink at specified intervals.

Observations and Inferences: Note if LED blinks as intended, adjust timing if needed.

Wokwi Lab-1 @ project

Lab-2

Lab Exercise: Blinking an External Red LED

Objective: Interface an external LED with Arduino and control it through a digital output pin.

Components: Arduino Uno, external LED, jumper wires.

#define vk 12

void setup() {

pinMode(vk, OUTPUT);

}

void loop() {

digitalWrite(vk, HIGH);

delay(500);

digitalWrite(vk, LOW);

delay(500);

}

Procedure:

- Connect the external LED to a digital pin.

- Write a program to blink the external LED.

- Upload code, observe the LED behaviour.

Expected Outcome: LED should blink on and off at specified intervals.

Observations and Inferences: Measure and note LED brightness

Wokwi Lab-2 @ Sketch

Lab-3

Lab Exercise: Reading & Displaying Humidity and Temperature Data

Objective: Read temperature and humidity data using a DHT11/DHT22 sensor.

Components: Arduino Uno, DHT sensor, breadboard, jumper wires.

Procedure:

- Connect the DHT sensor to Arduino.

- Install the DHT library and write code to display temperature and humidity.

- Upload the code and observe the serial output.

#include <DHT.h>

#define vk 7

#define DHTTYPE DHT22

DHT dht(vk, DHTTYPE);

float humid, temp;

void setup() {

Serial.begin(9600);

dht.begin();

}

void loop() {

delay(200);

humid = dht.readHumidity();

temp = dht.readTemperature();

Serial.print("Humidity: ");

Serial.print(humid);

Serial.print(" %  Temperature: ");

Serial.print(temp);

Serial.println("°C");

delay(1000);

}

Expected Outcome: Display accurate temperature and humidity data.

Observations and Inferences: Record the readings and note any variations in data.

wokwi Lab-3 @ Sketch

Lab-4

Lab Exercise: Configuring MQTT in our Local Machine

Objective: Explore the Mosquitto MQTT protocol and understand its functionalities.

In SystemOS [ Windows11 ]:

Install Mosquitto:

- Download and install Mosquitto as a service from the official Eclipse page.

- This allows the broker to start automatically in the background at boot.

Add Mosquitto to System PATH:

- Go to System Environment Variables and add the path C:\Program Files\mosquitto to allow running Mosquitto commands directly from the Command Prompt or Terminal.

Control Mosquitto Service:

- To start Mosquitto at boot: Use net start mosquitto.

- To stop Mosquitto manually in an elevated Command Prompt: Use net stop mosquitto.

Using Mosquitto in Command Prompt:

- Open Command Prompt and navigate to Mosquitto’s installation directory:

cd "C:\Program Files\mosquitto"

Run Mosquitto in Verbose Mode to view detailed logs and backend processes for debugging:

mosquitto.exe -v

This setup allows you to control the Mosquitto broker and see real-time logs, which is helpful for debugging and monitoring.

Procedure:

- Read about the Mosquitto MQTT protocol.

- Document its components (broker, client), setup, and messaging.

- Create a basic publisher-subscriber model using Mosquitto.

Expected Outcome: Knowledge of MQTT fundamentals.

Observations and Inferences: Note how Mosquitto is used for IoT communications.

Lab-5

Lab Exercise: Node-Red Web App for DHT Data

Objective: Build a web dashboard in Node-Red to display DHT sensor data.

Components: Node-Red environment, Arduino with DHT sensor.

Procedure:

- Set up Node-Red and connect Arduino with DHT data.

- Use the Node-Red dashboard nodes to display data.

- Test and document the data on the web interface.

Expected Outcome: Real-time temperature and humidity data on a web dashboard.

Observations and Inferences: Document data refresh rate and layout effectiveness.

Serial In Node: Configure it to read from the correct serial port where                                      your Arduino is connected (e.g., COM5). Set the baud rate to 9600.

  - Add the node and name it "Arduino".

Function Node: Add a function node and insert the following                                                     code to process the incoming serial data:

Debug Node: Connect it to the output of the serial node                                                               to check if data is flowing correctly.

Gauge Nodes:

Add two gauge nodes, one for humidity and another for temperature.

  - Humidity Gauge:

    - Set the title to "Humidity".

    - Set the value format to {{value}}%.

    - Set the minimum value to 0 and the maximum to 100.

  - Temperature Gauge:

    - Set the title to "Temperature".

    - Set the value format to {{value}}ºC.

    - Set the minimum value to 0 and the maximum to 100.

The flow is structured as follows:

- Serial In -> Function Node -> Humidity Gauge

- Serial In -> Function Node -> Temperature Gauge

Deploy and Test

- Deploy the flow in Node-RED.

- Open the Node-RED dashboard (usually available at http://<your-IP>:1880/ui).

- Check the Gauges: You should see live humidity and temperature readings from the DHT11/22 sensor displayed in the dashboard.

Global Variables and Objects:

DHT dht(DHTPIN, DHTTYPE); // Initialize DHT sensor

- DHT: This is the name of the class provided by the library. It's a blueprint for creating objects that represent DHT sensors in your code.

- dht: This is the name we're giving to the instance of the DHT class you're creating. It's like a variable that you'll use to interact with the DHT sensor.

- (DHTPIN, DHTTYPE): These are arguments we're passing to the constructor of the DHT class.

  - DHTPIN is a constant with the value , which tells the library which pin on the Arduino the data line of the sensor is connected to.

  - DHTTYPE is another constant with the value DHT11/22, which tells the library which type of DHT sensor you're using (the library also supports the DHT22 sensor).

Lab-6

Lab Exercise: HC-SR04 Ultrasonic Sensor

Objective: Measure distance using an ultrasonic sensor.

Components: Arduino, ultrasonic sensor, breadboard, jumper wires.

Procedure:

- Connect the ultrasonic sensor to Arduino.

- Write code to calculate and display distance based on echo time.

- Upload code and test for different objects and distances.

Expected Outcome: Accurate distance measurements on the serial monitor.

Observations and Inferences: Note sensor accuracy and limitations at varying distances.

Wokwi Lab-6 @ Sketch

Lab-7

Lab Exercise: Use of Breadboard

Objective: Learn basic breadboard interfacing and circuit assembly.

Components: Breadboard, jumper wires, resistors, LEDs.

In this project, we did the process to blink three LEDs using for loop. The three LEDs will light up one after the other.

Expected Outcome: Familiarity with breadboard layout and component arrangement.

Wokwi Lab-7 @ Sketch

Lab-8

Lab Exercise: ESP32 for Onboard LED Blink

Objective: Use the ESP32 microcontroller to blink its onboard LED.

Components: ESP32 board, Arduino IDE.

Procedure:

1. Install ESP32 Support in Arduino IDE:

- Go to File > Preferences in the Arduino IDE.

- In Additional Boards Manager URLs, paste: https://dl.espressif.com/dl/package_esp32_index.json and click OK.

- Open Boards Manager (Tools > Board > Boards Manager) and search for ESP32 by Espressif Systems. Click Install.

2.  Install USB Driver:

- Download and install the CP210x USB-to-UART Bridge VCP Driver to enable communication between your computer and the ESP32 board.

3. Select Board and Port in Arduino IDE:

- After restarting, select ESP32 Dev Module from the Tools > Board menu.

- Choose the correct COM port for your ESP32 (e.g., COM7).

4. Put ESP32 in Boot Mode:

- Press and hold the BOOT button while uploading the code.

- Release the BOOT button once the upload begins (when you see "Connecting...").

5. Code Execution for LED Blink:

- Use the following code in the Arduino IDE to blink the onboard LED.

Expected Outcome: ESP32 onboard LED blinks at specified intervals.